[image: image15.png]



SAFETY DIRECTIVE

Title:
GHS-Hazard Communication Program

Issuing Department:
Town Manager’s Office


Effective Date:
December 17, 2013

Reviewed:
September 01, 2014

Approved:
Gilbert Davidson, Town Manager

Type of Action:
Updated
1.0 PURPOSE

The Town of Marana has established this Hazard Communication Program to ensure that information about the dangers of all hazardous chemicals used by the Town is known by all affected employees. Under this Program, employees will be informed of the contents of the Occupational Safety and Health Administration (OSHA) Globally Harmonized System (GHS)-Hazard Communications Standard (HCS), the hazardous properties of chemicals with which employees work, safe handling procedures, and measures to take to protect employees from these chemicals.

2.0 DEPARTMENTS AFFECTED
This Program applies to all work operations in the Town where employees may be exposed to hazardous chemicals under normal working conditions or during an emergency situation. All work units of the Town will participate in the GHS-Hazard Communication Program.
3.0 REFERENCES
3.1 GHS-Hazard Communication Standard (HCS), 29 CFR 1910.1200
3.2 Town of Marana Personnel Policies and Procedures, Chapter 7, Safety and Health

4.0 DEFINITIONS
4.1 Chemical: Any element, chemical compound or mixture of elements and/or compounds.
4.2 Combustible liquid: Any liquid having a flashpoint at or above 100°F (37.8C), but below 200°F (93.3C), except any mixture having components with flashpoints of 200°F (93.3C), or higher.
4.3 Compressed gas:  A gas, a mixture of gases, or a liquid that falls into one of the following categories:
4.3.1 A gas or mixture of gases having, in a container, an absolute pressure exceeding 40 psi at 70° F (21.1C).
4.3.2 A gas or mixture of gases having, in a container, an absolute pressure exceeding 104 psi at 130°F (54.4C) regardless of the pressure at 70°F (21.1C).
4.3.3 A liquid having a vapor pressure exceeding 40 psi at 100°F (37.8C) as determined by ASTM D-323-72.

4.4 Container: Any bag, barrel, bottle, box, can, cylinder, drum, storage tank, or the like that contains a hazardous chemical. For purposes of this directive, pipes or piping systems, and engines, fuel tanks, or other operating systems in a vehicle, are not considered to be containers.
4.5 Explosive: A chemical that causes a sudden, almost instantaneous release of pressure, gas, and heat when subjected to sudden shock, pressure, or high temperature.
4.6 Exposure or exposed: When an employee is subjected to a hazardous chemical in the course of employment through any route of entry (inhalation, ingestion, skin contact or absorption, etc.), and includes potential (e.g. accidental or possible) exposure.
4.7 Flammable: A chemical that falls into one of the following categories:
4.7.1 An aerosol that, when tested by the method described in 16 CFR 1500.45, yields a flame projection exceeding 18 inches at full valve opening, or a flashback (a flame extending back to the valve) at any degree of valve opening.

4.7.2 A gas that, at ambient temperature and pressure, forms a flammable mixture with air at a concentration of 13 percent by volume or less.

4.7.3 A gas that, at ambient temperature and pressure, forms a range of flammable mixtures with air wider than 12 percent by volume, regardless of the lower limit.

4.7.4 Any liquid having a flashpoint below 100°F (37.8C), except any mixture having components with flashpoints of 100°F (37.8C) or higher, the total of which make up 99 percent or more of the total volume of the mixture.

4.7.5 A solid, other than a blasting agent or explosive as defined in 29 CFR 1910.109(a), that is liable to cause fire through friction, absorption of moisture, spontaneous chemical change, or retained heat from manufacturing or processing, or which can be ignited readily and when ignited burns so vigorously and persistently as to create a serious hazard. A chemical shall be considered to be a flammable solid if, when tested by the method described in 16 CFR 1500.44, it ignites and burns with a self-sustained flame at a rate greater than one-tenth of an inch per second along its major axis.

4.8 Flashpoint: The minimum temperature at which a liquid gives off a vapor in sufficient concentration to ignite.

4.9 Globally Harmonized System (GHS): An international approach to hazard communication, providing agreed criteria for classification of chemical hazards, and a standardized approach to label elements and safety data sheets. The GHS provides: 

4.9.1 Harmonized classification criteria for health, physical, and environmental hazards of chemicals. 

4.9.2 Standardized label elements that are assigned to these hazard classes and categories, and provide the appropriate signal words, pictograms, and hazard and precautionary statements to convey the hazards to users. 

4.9.3 A standardized order of information for Safety Data Sheets. 
4.10 Hazard warning: Any words, pictures, symbols, or combination thereof appearing on a label or other appropriate form of warning which convey the hazard(s) of the chemical(s) in the container(s).

4.11 Hazardous chemical: Any chemical that is a physical hazard or a health hazard.

4.12 Health hazard: A chemical for which there is statistically significant evidence that acute or chronic health effects may occur in exposed employees. The term health hazard includes chemicals which are carcinogens, toxic or highly toxic agents, reproductive toxins, irritants, corrosives, sensitizers, hepatotoxins, nephrotoxins, and neurotoxins, agents which act on the hematopoietic system and agents which damage the lungs, skin, eyes, or mucous membranes.

4.13 Immediate use: When the hazardous chemical will be under the control of and used only by the person who transfers it from a labeled container and only within the work shift in which it is transferred.

4.14 Oxidizer: A chemical, other than a blasting agent or explosive as defined in 29 CFR 1910.109(a), that initiates or promotes combustion in other materials, thereby causing fire either of itself or through the release of oxygen or other gases.

4.15 Physical hazard: A chemical for which there is scientifically valid evidence that it is a combustible liquid, a compressed gas, explosive, flammable, an organic peroxide, an oxidizer, pyrophoric, unstable (reactive) or water-reactive.

4.16 Pyrophoric: A chemical that will ignite spontaneously in air at a temperature of 130°F (54.4C) or below.

4.17 Safety Committee:  A committee composed of Town employees with the overall goals of assisting the Town in monitoring, improving, and maintaining job site safety and occupational health as mandated by OSHA regulations, and in achieving and maintaining a safe and healthy workplace for Town employees.
4.18 Safety Coordinator: The Town employee responsible for all aspects of the Town’s safety program, or his or her designee. In the absence of a Safety Coordinator, the Town Manager may designate another employee(s) or a contractor(s) to perform the functions of the Safety Coordinator.

4.19 Safety Data Sheet (SDS): Written or printed material concerning a hazardous chemical which is prepared in accordance with 29 CFR 1910.1200(g).

4.20 Unstable (reactive): A chemical which in the pure state, or as produced or transported, will vigorously polymerize, decompose, condense, or will become self-reactive under conditions of shocks, pressure or temperature.

4.21 Water-reactive: A chemical that reacts with water to release a gas that is either flammable or presents a health hazard; often, when the water is heated, it goes into a gaseous state allowing oxygen to be released which can help feed a fire.
5.0 POLICIES AND PROCEDURES
5.1 Written Program. The Town of Marana shall maintain this safety directive as its written GHS-Hazard Communication Program in accordance with 29 CFR 1910.1200.  
5.1.1 The Town shall annually review and revise this Program based on the Town's operational requirements, or as required by the OSHA GHS-Hazard Communication Standard.  
5.1.2 Paper copies of this GHS-Hazard Communication Program shall be available in every department throughout the Town.  The Program shall also be available on the Town employee intranet.
5.1.3 The written Program shall:

5.1.3.1 Be communicated to all personnel who are affected by it. 
5.1.3.2 Provide a system for proper labeling of containers, describe other needed forms of warning, and detail the use and purpose of SDS. 
5.1.3.3 Describe how employee information and training requirements will be met.  

5.1.3.4 Be made available to all employees during each work shift.
5.2 Training Program. The Town of Marana shall provide employees with information and training on hazardous chemicals in their work area at the time of their initial assignment, at least annually, and whenever a new chemical is introduced into their work area that could present a potential hazard.
5.2.1 All employees who work with or are potentially exposed to hazardous chemicals will receive initial training on the GHS-Hazard Communication Standard and this Program before starting work and annually thereafter. Each new employee will attend a health and safety orientation that includes the following information and training:
5.2.1.1 An overview of the OSHA GHS-Hazard Communication Standard 

5.2.1.2 The hazardous chemicals present at his/her work area 

5.2.1.3 The physical and health risks of the hazardous chemicals 

5.2.1.4 Symptoms of overexposure 

5.2.1.5 How to determine the presence or release of hazardous chemicals in the work area 

5.2.1.6 How to reduce or prevent exposure to hazardous chemicals through use of control procedures, work practices and personal protective equipment 

5.2.1.7 Steps the Town has taken to reduce or prevent exposure to hazardous chemicals 

5.2.1.8 Procedures to follow if employees are overexposed to hazardous chemicals 

5.2.1.9 How to read labels and SDS to obtain hazard information 

5.2.1.10 Location of the SDS file or book and written GHS-Hazard Communication Program 

5.2.2 Prior to introducing a new chemical hazard into any work area at the Town, each employee in that work area will be given information and training as outlined above for the new chemical hazard. The training may be accomplished via classroom instruction, interactive computer programs, audiovisuals, safety training videos, or other effective methods.

5.2.3 Employees transferred to another Town department either temporarily or permanently will be briefed on the duty-specific hazards by their immediate supervisor before they begin any duties within the department. Emergency and first aid procedures will be included in the training.

5.2.4 Training shall be documented. A copy of the attendance sign-in sheet or the copy of the quiz (if applicable) shall be maintained as part of the employee’s personnel file.

5.3 Container Labeling.  All containers that contain materials covered under this Program shall be properly labeled. The Safety Coordinator or designee will review the Town’s labeling procedures on an annual basis and will update labels as required.

5.3.1 All containers received for use shall be clearly labeled as to the contents, shall note the appropriate hazard warning, and shall list the manufacturer's name and address.

5.3.2 All secondary containers shall be labeled with either an extra copy of the original manufacturer's label or with labels marked with the identity and the appropriate hazard warning. 

5.3.3 On the following individual stationary process containers, the Town uses placards and posts Safety Data Sheets in the area, rather than, or in addition to, a label to convey the required information:

	Stationary Process Container
	Location

	Uline H-22195 Flammable Storage Cabinet
	Fleet Shop - South Wall

	Accu tabs – calcium hypochlorite tabs
	Airport pump house and trucks

	Round-up 30 gallon containers
	Public Works yard


5.3.4 Unmarked containers containing chemicals may not be used in conjunction with any duties or operations, unless the container is a portable container in the control of a specific person for their immediate use.  

5.3.5 The GHS-Hazard Communication Standard does not require labeling of the following chemicals:

5.3.5.1 Any pesticide as such term is defined in the Federal Insecticide, Fungicide, and Rodenticide Act (7 U.S.C. § 136 et seq.), when subject to the labeling requirements of that Act and labeling regulations issued under that Act by the Environmental Protection Agency.

5.3.5.2 Any food, food additive, color additive, drug, cosmetic, or medical or veterinary device, including materials intended for use as ingredients in such products (e.g. flavors and fragrances), as such terms are defined in the Federal Food, Drug, and Cosmetic Act (21 U.S.C. § 301 et seq.) and regulations issued under that Act, when they are subject to the labeling requirements under that Act by the Food and Drug Administration.
5.3.5.3 Any consumer product or hazardous substance as those terms are defined in the Consumer Product Safety Act (15 U.S.C. § 2051 et seq.) and Federal Hazardous Substances Act (15 U.S.C. § 1261 et seq.) respectively, when subject to a consumer product safety standard or labeling requirement of those Acts, or regulations issued under those Acts by the Consumer Product Safety Commission.

5.3.5.4 Labeling of containers of chemicals and hazardous materials being shipped off site designated as hazardous waste.  Where these materials are classified as hazardous waste, they fall under the Solid Waste Disposal Act, as amended by the Resource Conservation and Recovery Act of 1976, as amended (42 U.S.C. § 6901 et seq.), and the provisions of 40 CFR, and as such will be subject to regulations issued under that Act by the Environmental Protection Agency.  

5.4 Safety Data Sheets (SDS).  The Town shall establish an SDS program in compliance with the GHS-Hazard Communication Standard.  The Town shall ensure that procedures are developed to obtain the necessary SDS, to review incoming SDS for new or significant health and safety information, and to communicate any new information to affected employees. 
5.4.1 Copies of SDS for all hazardous chemicals to which employees are exposed or are potentially exposed will be kept, at a minimum, in the locations listed on Attachment C.
5.4.2 SDS will be readily available to all employees during each work shift. If an SDS is not available, the employee should contact his/her immediate supervisor, the Department Head or the Safety Coordinator.

5.4.3 When revised SDS are received, the following procedures will be followed to replace old SDS:  

5.4.3.1 The Safety Coordinator or designee shall send an email to all affected employees advising them of the new, revised SDS within five work days of receipt of the new SDS.

5.4.3.2 The Safety Coordinator or designee will send an email notification to all Safety Committee members advising them to update the SDS books located in their areas.

5.4.4 If an SDS is not received at the time of an initial shipment of a chemical, the employee receiving the chemical shall call the vendor and request the SDS. The employee shall make the call within one day of receipt of the material.  

5.5 Globally Harmonized System (GHS).  Hazard Identification (Section 2) of the SDS and the chemical container label (reference Attachment A) contain Hazard Identification information that shall be conveyed to the employee prior to chemical use.  Hazard Identification comes in the form of:

5.5.1 Pictograms (see figures below)
5.5.2 Hazard Statements

5.5.3 Precautionary Statements
	Description
	Pictogram
	Hazard class and hazard category:


[image: image1.png]2

=
MARANA AZ




Exploding Bomb
Unstable explosives
Explosives of Divisions 1.1, 1.2, 1.3, 1.4
Self-reactive substances and mixtures, Types A, B Organic peroxides, Types A, B
[image: image7.jpg]


[image: image8.jpg]


Flame
Flammable gases, category 1
Flammable aerosols, categories 1,2 Flammable liquids, categories 1,2,3 Flammable solids, categories 1,2
Self-reactive substances and mixtures, Types B,C,D,E,F Pyrophoric liquids, category 1
Pyrophoric solids, category 1
Self-heating substances and mixtures, categories 1,2 Substances and mixtures, which in contact with water, emit flammable gases, categories 1,2,3
[image: image9.jpg]


Organic peroxides, Types B,C,D,E,F
[image: image10.jpg]


Flame Over Circle
Oxidizing gases, category 1 Oxidizing liquids, categories 1,2,3
[image: image11.jpg]


[image: image12.jpg]


Gas Cylinder
Gases under pressure:
· Compressed gases
· Liquefied gases
· Refrigerated liquefied gases
· Dissolved gases
[image: image13.jpg]


[image: image14.jpg]


Corrosion
Corrosive to metals, category 1
Skin corrosion, categories 1A, 1B, 1C Serious eye damage, category 1
Skull and Crossbones
[image: image2.jpg]


   Acute toxicity (oral, dermal, inhalation), categories 1, 2, 3
Exclamation Mark
Acute toxicity (oral, dermal, inhalation), category 4 Skin irritation, category 2
Eye irritation, category 2  Skin sensitization, category 1
Specific Target Organ Toxicity – Single exposure, category 3

Health Hazard

Respiratory sensitization, category 1
Germ cell mutagenicity, categories 1A,1B,2 Carcinogenicity, categories 1A,1B,2 Reproductive toxicity, categories 1A,1B,2
Specific Target Organ Toxicity – Single exposure, categories 1, 2 Specific Target Organ Toxicity – Repeated exposure, categories 1,2

Aspiration Hazard, category 1
Environment

     Hazardous to the aquatic environment


     Acute hazard, category1


     Chronic hazard, categories 1,2
5.5.4 HMIS Color Bar.  The HMIS Color Bar is similar to the NFPA fire diamond. The color bar is not for emergencies and is used to convey broader health warning information.

[image: image3.png]Chemical Name

HEALTH

0

0
PERSONAL PROTECTION | O





5.5.4.1 The four bars are color-coded, using the modern color bar symbols with blue indicating the level of health hazard, red for flammability, orange for a physical hazard, and white for Personal Protection. The number ratings range from 0-4.

5.5.4.2 The Blue/Health section conveys the health hazards of the material. If present, an asterisk signifies a chronic health hazard, meaning that long-term exposure to the material could cause a health problem.  The number ratings mean the following:
4  Life-threatening, major or permanent damage may result from single or repeated overexposures.
3  Major injury likely unless prompt action is taken and medical treatment is given.
2  Temporary or minor injury may occur.

1  Irritation or minor reversible injury possible.

0  No significant risk to health

5.5.4.3 The Red/Flammability section conveys the flammability of the material.  For HMIS 0, 1 and 2, the criteria used to assign numeric values are identical to those used by NFPA. For HMIS 3 and 4, the flammability criteria are defined according to OSHA standards.  The number ratings mean the following:
4 Flammable gases, or very volatile flammable liquids with flash points below 73 °F (23 °C), and boiling points below 100 °F (38 °C). Materials may ignite spontaneously with air.

3 Materials capable of ignition under almost all normal temperature conditions. This includes flammable liquids with flash points below 73 °F (23 °C) and boiling points above 100 °F (38 °C), as well as liquids with flash points between 73 °F and 100 °F.

2 Materials which must be moderately heated or exposed to high ambient temperatures before ignition will occur. This includes liquids having a flash point at or above 100 °F (38 °C) but below 200 °F (93°C).
1
Materials that must be preheated before ignition will include liquids, solids and semi solids having a flash point above 200 °F (93 °C).

0 Materials that will not burn.

5.5.4.4 The Orange/Physical Hazard section conveys the physical hazard of the material. Reactivity hazards are assessed using the OSHA criterion of physical hazard, as follows:
4
Materials that is readily capable of explosive water reaction, detonation or explosive decomposition, polymerization, or self- reaction at normal temperature and pressure.
3
Materials that may form explosive mixtures with water and are capable of detonation or explosive reaction in the presence of a strong initiating source. Materials may polymerize, decompose, self-react, or undergo other chemical change at normal temperature and pressure with moderate risk of explosion.
2
Materials that are unstable and may undergo violent chemical changes at normal temperature and pressure with low risk for explosion. Materials may react violently with water or form peroxides upon exposure to air.
1
Materials that are normally stable but can become unstable (self- react) at high temperatures and pressures. Materials may react non-violently with water or undergo hazardous polymerization in the absence of inhibitors.

0
Materials that are normally stable, even under fire conditions, and will not react with water, polymerize, decompose, condense, or self-react non-explosives.

5.5.4.5 The White/Personal Protection section conveys information regarding what personal protective equipment (PPE) should be used when working with the material by assigning a letter(s) A through X.  See figure below.
[image: image4.jpg]HAZARDOUS IATERIALS IDENTIEICATIONISY ST

4= SEVERE HAZARD
3=SERIOUS HAZARD

2=MODERATE HAZARD
1= SLIGHT HAZARD
0= MINIMAL HAZARD

nasers oo
dsgrion oregonds
oatitonlomaion
nadasshedor st
o lesmilizicn

G ermator

7N

PN+ -

Eal 2 )

5+ e + &

)

PN ek

T

=R 28 5]

TN+ e+ 5

L

o R

Cosatyourspentorar.05 v SSECAL eningaclons





5.6 Other Employers/Contractors.  The Safety Coordinator or designee will provide other employers and contractors with information about hazardous chemicals that their employees may be exposed to on a job site and suggested precautions for employees. The Safety Coordinator or designee will also obtain information about hazardous chemicals used by other employers and contractors to which employees of the Town may be exposed.

5.6.1 The Town will provide other employers and contractors working onsite with copies of relevant SDS for hazardous chemicals used by the Town.
5.6.2 In addition to providing a copy of an SDS to other employers and contractors, other employers and contractors will be informed of necessary precautionary measures to protect employees exposed to operations performed by the Town.  In some circumstances, the Town may provide a safety orientation for contractors or other employers before they begin work onsite, to review hazards of the job to be performed, as well as the Town’s safety policies and/or procedures.

5.7 Hazardous Non-routine Tasks. Periodically, employees are required to perform non‑routine tasks that involve the use of or exposure to hazardous chemicals. Examples of non-routine tasks are: confined space entry, tank cleaning, and painting reactor vessels. Prior to starting work on such projects, each affected employee will be given information by the Safety Coordinator or designee, or the immediate supervisor, about the hazardous chemicals the employee may encounter during the task. This information will include specific chemical hazards, protective and safety measures the employee should use, and steps the Town is taking to reduce the hazards, including ventilation, respirators, the presence of another employee (buddy systems), and emergency procedures.  
5.7.1 The following are examples of non-routine tasks performed by Town employees:
	Task
	Hazardous Chemical

	Clean up spills on roads from accidents, etc.
	Motor oil, anti-freeze, transmission fluid

	Paint inside of storage tanks
	Bar-rust 233 multi-purpose epoxy coating

	Application of pesticides
	Herbicides, insecticides, post- and pre-emergent

	Confined space entry
	Methane and hydrogen sulfide

	Changing batteries
	Battery acid

	Storm water testing
	Various chemicals in test kits

	Disinfection of storage tanks
	Sodium hypochlorite


5.8 List of Hazardous Chemicals. A list of all known hazardous chemicals used by Town employees is located on the Town employee intranet. This list includes the name of the chemical or product, common name, the manufacturer, the SDS revision date, the hazard rating for each chemical or product, if available, and the location in which the chemical is used. Further information on each chemical or product may be obtained from the specific SDS, located in the individual SDS binders throughout Town facilities.
5.8.1 When new chemicals are received, this list shall be updated within 30 days by the Safety Coordinator or designee. To ensure any new chemical is added in a timely manner, the following procedures shall be followed:

5.8.1.1 The employee who receives the new chemical shall report the receipt of the chemical to the employee’s foreman or supervisor.

5.8.1.2 The foreman or supervisor shall report the receipt of the new chemical to the Safety Coordinator or designee.
5.8.2 The hazardous chemical inventory is compiled and maintained by the Safety Coordinator or designee.

5.9 Chemicals in Unlabeled Pipes. When work activities are performed by employees in areas where chemicals are transferred through unlabeled pipes, prior to starting work, the employee shall contact the immediate supervisor, the Department Head or the Safety Coordinator for information regarding: 

5.9.1 The chemical in the pipes 

5.9.2 Potential hazards 

5.9.3 Required safety precautions 

5.10 Program Availability.  A copy of this GHS-Hazard Communication Program will be made available, upon request, to employees and their representatives.

6.0 RESPONSIBILITIES

6.1 The Safety Coordinator is the Program coordinator, with overall responsibility for the Program.  The Safety Coordinator is responsible for facilitating development, review, and revision of this GHS-Hazard Communication Program. The Safety Coordinator shall consult with the Town Manager regarding decisions necessary to ensure the success of this Program, and regarding appropriate changes and amendments to this safety directive. 

6.2 Department Heads, managers and supervisors are responsible for ensuring that the requirements of this Program are fully implemented in their work areas.

6.3 Employees are responsible for attending all mandatory training classes, and understanding the policies and procedures outlined in this Program, as well as all Town health and safety procedures. 
6.4 The Safety Coordinator and the Safety Committee are authorized to halt any operation of the Town where there is danger of serious personal injury.  
7.0 ATTACHMENTS

7.1 Attachment A - Hazard Information/Chemical Shipping Label and Interpretation

7.2 Attachment B – OSHA Quick Card: Hazard Communication Safety Data Sheets
7.3 Attachment C – Safety Data Sheet Book Locations 
7.4 Attachment D - Master Chemical Inventory (Pages 1-24)
Attachment A 

HAZARD INFORMATION/ CHEMCIAL SHIPPING LABEL AND INTERPRETATION
[image: image5.jpg]Wegnt ‘
Benzene « DKy 1. Name of Chemical
\mmmulmlumnmn—““‘“‘—‘—w No. 1114 2. Barcode
Hazard < 3. Precautionary
Statement

DO

e —
o - < 5. Hazard Information

28t i s )y o e e s nd o

e oo et s ks e e e 6. Note of caution

inaind ot s e sncrors

P E T
o
B,

Tt Yooet oy ot o e S T
PTELTD 7. Manufacturer or

Provider of

Chemical company





Attachment B
[image: image6.jpg]OSHA 46




Hazard Communication Safety Data Sheets
The Hazard Communication Standard (HCS) requires chemical manufacturers, distributors, or importers to provide Safety Data Sheets (SDSs) (formerly known as Material Safety Data Sheets or MSDSs) to communicate the hazards of hazardous chemical products. As of June 1, 2015, the HCS will require new SDSs to be in a uniform format, and include the section numbers, the headings, and associated information under the headings below:
Section 1, Identification includes product identifier; manufacturer or distributor name, address, phone number; emergency phone number; recommended use; restrictions on use.
Section 2, Hazard(s) identification includes all hazards regarding the chemical; required label elements.
Section 3, Composition/information on ingredients includes information on chemical ingredients; trade secret claims.
Section 4, First-aid measures includes important symptoms/ effects, acute, delayed; required treatment.
Section 5, Fire-fighting measures lists suitable extinguishing techniques, equipment; chemical hazards from fire.
Section 6, Accidental release measures lists emergency procedures; protective equipment; proper methods of containment and cleanup.
Section 7, Handling and storage list precautions for safe handling and storage, including incompatibilities.
Section 8, Exposure controls/personal protection lists OSHA's Permissible Exposure Limits (PELs); Threshold Limit Values (TLVs); appropriate engineering controls; personal protective equipment (PPE).
Section 9, Physical and chemical properties lists the chemical's characteristics.
Section 10, Stability and reactivity list chemical stability and possibility of hazardous reactions.
Section 11, Toxicological information includes routes of exposure; related symptoms, acute and chronic effects; numerical measures of toxicity.
Section 12, Ecological information* 
Section 13, Disposal considerations* 
Section 14, Transport information* 
Section 15, Regulatory information*
Section 16, Other information, includes the date of preparation or last revision.

ATTACHMENT C - SAFETY DATA SHEET (SDS) BOOK LOCATIONS
MARANA MUNICIPAL COMPLEX (MMC), 11555 W. CIVIC CENTER DR.
	FLOOR/BUILDING
MMC 1ST FLOOR EAST
	LOCATION
COPIER AREA
	DEPARTMENT
NEAR TOWN CLERK’S OFFICE

	MMC 2ND FLOOR EAST MMC 2ND FLOOR WEST MMC 3RD FLOOR EAST MMC 3RD FLOOR WEST POLICE HEADQUARTERS
	KITCHEN AREA KITCHEN/COPIER AREA RECEPTION AREA KITCHEN/FILE AREA
2ND FLOOR NEAR CHIEF’S OFFICE
	PLANNING/DEVELOPMENT SERVICES ENGINEERING
TOWN MANAGER’S OFFICE HUMAN RESOURCES

POLICE

	MUNICIPAL COURT
	EMPLOYEE BREAK ROOM
	COURT


MARANA OPERATIONS CENTER (MOC), 5100 W. INA RD.
DEPARTMENT                                             LOCATION
UTILITIES                                                     KITCHEN AREA
POLICE SUBSTATION                                PROPERTY AND EVIDENCE PROCESS ROOM

PUBLIC WORKS                                          DIRECTOR’S OFFICE, COMMUNITY RESPONSE MANAGER’S OFFICE, SIGN SHOP, TOOL

SHED, ASPHALT MOBILE MINI, TRUCK 289
OTHER LOCATIONS
DEPARTMENT                                             LOCATION
PARKS AND RECREATION                       EQUIPMENT/MAINTENANCE YARD, ORA MAE HARN PARK, 13250 N. LON ADAMS RD. PARKS AND RECREATION                       MAIN OFFICE, BY COPIER, ADMINISTRATION BUILDING, 13251 N. LON ADAMS RD. PARKS AND RECREATION                       COPY ROOM, COMMUNITY CENTER, 13250 N. LON ADAMS ROAD

MARANA REGIONAL AIRPORT              DIRECTOR’S OFFICE, 11700 W. AVRA VALLEY RD.

UTILITIES/WASTEWATER DIVISION     OFFICE, MARANA WASTEWATER RECLAMATION FACILITY, 14393 N. LUCKETT RD.

Department

Attachment D - Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE
	Parks-Admin
	Expo Cleaner for Dry Erase Surfaces
	Dry Erase Cleaner
	Sanfor Corporation
	6/1/2001
	2
	2
	0
	0

	
	Goof Off Professional Strength
	Goof Off
	W.M. Barr
	6/28/2011
	2
	3
	0
	0

	
	Hot Shot Fogger
	Fogger
	Chemisco
	7/16/2008
	1
	2
	0
	P

	
	Oxy-Steam Carpet Cleaner
	Carpet Cleaner
	Rug Doctor
	1/31/2012
	0
	0
	0
	0

	
	Black Flag Ant & Roach Killer
	Black Flag
	Chemisco
	9/20/2012
	1
	2
	0
	P

	Parks-Facilities
	Century Duster
	Century Duster
	Falcon Safety Products
	10/28/2009
	1
	4
	1
	x

	
	Expo Cleaner for Dry Erase Surfaces
	Dry Erase Cleaner
	Sanfor Corporation
	6/1/2001
	2
	2
	0
	0

	
	Insecticide
	Dusted
	United laboratories
	3/25/2008
	2
	1
	0
	0

	
	Insecticide
	RID
	Continental Research Corp
	3/26/2004
1
1
0
P

	
	Graffiti Remover
	Graffiti Be Gone
	Continental Research Corp
	1/30/2002
2
1
0
0

	
	Graffiti Remover
	Graffiti Gone
	Continental Research Corp
	11/15/2005
1
3
1
P

	
	
	GOJO Luxury Foam
	
	

	
	Hand wash
	Hand wash
	GOJO Industries
	12/27/2006
1
0
0
0

	
	Cleaner
	Flash
	Continental Research Corp
	1/23/2006
2
3
0
0

	
	Cleaner
	Consume Eco-Lyzer
	Spartan Chemical Co
	6/30/2005
3
0
0
?

	Parks-Pool
	
	
	
	
	
	
	
	

	Barkeepers Friend Cleanser & Polish
Barkeepers Friend
	SerVaas Laboratories, Inc.
	11/1/2012
2
0
0
P

	Bleach
Clorox Bleach
	Clorox
	7/12/11
3
0
1
E

	Hydrochloric Acid
Hasa Muriatic Acid
	Hasa Inc.
	8/21/2011
3
0
1
0

	Sodium Carbonate
Soda Ash
	Leslie's
	6/20/2008
2
0
0
B

	Glass Cleaner w/Ammonia
Windex Glass Cleaner
	SC Johnson
	12/5/2012
1
2
0
0

	Foaming Disinfectant Cleaner
Lysol Disinfectant
	Reckitt Benckiser
	11/7/2001
1
1
0
0

	Aqueous Acidic Cleaner
CLR
	AdVil Pty Ltd
	7/4/2011
1
0
0
P

	Parks-Community
Dawn Advanced Dish Soap
Dawn Dish soap
	Proctor & Gamble
	8/8/2006
1
1
0
0

	Center
Swiffer Wet Jet Antibacterial Cleaner
Floor Cleaner
	Proctor & Gamble
	12/1/2004
1
1
1
0

	Scrubbing Surface
	
	

	Kirkland Scrubbing Surface Wipes
Wipes
	Nice-Pak
	2/1/2005
0
2
0
0

	Kirkland Signature Surface Wipes
Surface Wipes
	Nice-Pak
	6/1/2003
0
2
0
0



Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE

	
	Lysol Brand Disinfectant Spray
	Lysol Spray
	Reckitt Benckiser
	7/9/2010
	1
	3
	0
	B

	Utilities-Field
	Acetylene
	Acetylene
	Paxair
	8/1/04
	0
	4
	2
	

	
	Alex Plus
	Latex Caulk
	DAP inc
	11/19/03
	1
	1
	0
	

	
	Aluminum Oxide
	Aluminum Oxide
	DYNA Systems
	11/19/03
	0
	0
	0
	

	
	Armor All
	Armor All
	Armor All
	01/01/98
	0
	0
	0
	

	
	Acetone
	Klean Strip Acetone
	WM Barr
	11/13/08
	1
	3
	0
	

	
	Brakleen Brake Cleaner
	Brake Cleaner
	CRC Industries
	01/27/03
	2
	3
	0
	

	
	Cement Treated Base
	CTB
	Granite
	9/21/02
	0
	0
	0
	

	
	Crafco Roadsaver
	Modified Asphalt
	Crafco inc
	8/13/07
	2
	1
	0
	

	
	Crack Filler
	Hot Applied crack filler
	Crack Filler corp.
	8/1/92
	1
	1
	0
	

	
	Cut Through Aerosol-Terpene
	Cut Through Aerosol
	Certified Labs
	8/29/07
	2
	4
	0
	

	
	Cutback Asphalt
	Cold Mix
	Granite
	8/21/02
	0
	0
	0
	

	
	Diesel w/ #2 Dye
	Diesel
	Chemtrec
	10/18/06
	1
	2
	0
	

	
	DD-135
	Sulonate mixture
	Chemsearch
	8/27/02
	2
	1
	0
	

	
	DUO Power-alkaline detergent
	Duo Power
	Chemsearch
	3/15/02
	2
	1
	0
	

	
	Durabond 70
	Epoxy
	Chemsearch
	9/8/04
	2
	1
	0
	

	
	Duraweld open arc
	welding wire
	X-Ergon
	8/9/01
	2
	0
	0
	

	
	Dylex PS
	Solvent
	Certified Labs
	4/27/00
	2
	4
	0
	

	
	Exalibur 7018 MR
	Covered Electrode
	Lincoln Electric Co.
	4/15/04
	1
	0
	0
	

	
	EZ Glide
	Aluminum Oxide
	Trust-X Partsmaster
	5/8/06
	1
	0
	0
	

	
	Fuel Additive
	Fuel Additive
	BG Products
	9/1/10
	2
	3
	0
	

	
	Flexible shrink tubing
	Polyolefins
	DYNA Systems
	4/11/01
	0
	0
	0
	

	
	Glue Redhot
	Redhot Glue
	Christys
	8/1/11
	2
	3
	1
	

	Green window glass
Glass Cleaner
cleaner
Berkley Packaging
5/4/11
0
	0
	0
	

	Gumout Starting Fluid
Starting Fluid
SOPUS
10/22/08
0
	2
	0
	

	Grease
White Litium Grease
BG Products
4/26/10
1
	3
	0
	

	GOJO hand cleaner
pumice hand cleaner
GOJO
4/27/11
1
	1
	0
	

	Gasoline
Gasoline
Chemtrec
1/8/04
1
	3
	0
	

	Graffiti-X
Graffiti-X
Champion Chemical
6/1/03
2
	2
	0
	

	Grinding and cutting wheels
cutting wheels
United Abrasives inc
11/1/03
1
	0
	0
	

	Hot Mix Asphalt
Hot Mix Asphalt
Granite
9/21/02
0
	0
	0
	


Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE
Keynote
Solvent
Drummond American corp.      5/1/01        2     0     0
Klean Strip thinner
paint thinner
WM Barr
12/14/05 1 2 0 Lubricant Ease-on pipe joint
Lubricant pipe joint
Seacord corp
2/1/06        0 0  0 Maxi Lube ultra #2
Maxi-Lube
Chemsearch
5/18/05        0     0     0

Magnesium Oxide
Ammonium sulfate/silica
Chemsearch
3/2/04        1     0     0
Mistoil Aerosol
Mistoil Aerosol
Chemsearch
12/30/04      1     4     0

Naturalizer Aerosol
Naturalizer Aerosol
Chemsearch
8/29/07    2     4     0 MP 2-CYCLE TC-W3 MOTOR OIL
2-CYCLE OIL
Valvoline
5/20/10       1     2   n/a
All Season Motor Oil 10W30
10w30 Motor Oil
Olympic Oil, LTD
6/15/99   1    1    0 SAE 15W-40 Motor Oil
15-40 Motor Oil

Parts Plus
5/7/10        1 1 0 2-Cycle Engine Oil
2-CYCLE OIL
Olympic Oil, LTD
2/13/08       1     1     0

Oxygen Compressed
Oxygen

Praxair, Inc.
9/1/04        0 0 0 Oatey Purple Primer/Cleaner
Purple Primer


Oatey
8/2/12        2 3 1 Based Inverted Marking Paint
Blue stake paint
Rust-Oleum Corporation   3/22/07   2   4    0 Marking Paint / Aerosols
Blue stake paint
Rust-Oleum Corporation  11/16/12   2   4   0 PN-105 Aerosol

Partsmaster, Div of NCH 12/21/07  2  4  0 Premalube
grease

Premalube
3/11/08 1  1  0 Marking Paint / Aerosols
Blue-Blue stake paint        Rust-Oleum Corporation      11/16/12      2     4     0

Rustlick WS-5050

ITW Fluid Products Group   5/30/02   1   1   0 Sealant
Great Stuff

Dow
1-37-08       1     3     0

Solder
solder on terminals
Dyna Systems
7/6/99
1
0
0 argon and carbon

Stargold

dioxideChemtrec
5/1/99
1
0
0
Tank Tonic
Tank Tonic
Certified Labs
12/16/93      1     1     1

Tekusolv aerosol
Tekusolv aerosol

Chemtrec Labs
4/14/08       1 4 0 Thermo Gloss-Urethane
Thermo Gloss
Communication stds
4/20/94       1 0 0 Thermo Strip floor finish
floor finish
Rochester midland
10/18/94   3   0   1 Ultra Solv 221
solvent


Chemsearch
7/22/05       1     1     0

WD40
Lubricant WD40
WD40
9/1/07        1     2     1

Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE
Weld-on
pipe thread sealant
IPS corp.
12/1/11
1
1
0

Yield Aerosol
Yield Aerosol
Chemsearch
12/21/07
2
4
0

Z-disk
Aluminum oxide
Trust-X Partsmaster
5/12/04
1
0
0

	Utilities-Admin
	Clorox Disinfecting Wipes - Fresh Scent
	Disinfecting wipes
	The Clorox Company
	5/11/2011
	N/A
	N/A
	N/A
	N/A

	
	3M Electronic Equipment Cleaner
	Electronic equipment
cleaner
	3M
	1/6/2012
	2
	2
	0
	N/A

	
	Office Depot Cleaning Duster
	Compressed air cleaner
	Amrep Automotive
Products Group
	10/9/2003
	1
	0
	0
	N/A

	
	Bic White-Out liquid
	White-out
	Bic USA
	N/A
	N/A N/A N/A
	N/A

	
	Liquid Paper White-Out
	White-out
	Sanford LP
	8/1/2005
	2
3
0
	N/A

	
	Liquid Paper Correction Tape
	White-out
	Newell Rubbermaid Office
Products
	8/3/2012
	N/A N/A N/A
	N/A

	
	Office Depot Refill Ink
	Refill ink
	Microcell Products
Company
	N/A
	N/A N/A N/A
	N/A

	
	Office Depot Highlighters
	Highlighters
	Newell Rubbermaid Office
Products
	3/16/2012
	N/A N/A N/A
	N/A

	
	Sharpie Permanent Markers
	Permanent markers
	Newell Rubbermaid Office
Products
	6/29/2011
	N/A N/A N/A
	N/A

	
	Expo Dry Erase Markers
	Dry erase markers
	Newell Rubbermaid Office
Products
	11/29/2012
	0
1
0
	N/A

	
	Berol Highlighters
	Highlighters
	Newell Rubbermaid Office
Products
	3/5/2013
	N/A N/A N/A
	N/A

	
	Counterfeit Detection Pen
	Counterfeit money
detector pen
	Dri Mark Products Inc.
	9/12/2011
	1
1
0
	A

	
	Expo Dry Erase Board Cleaner
	Whiteboard cleaner
	Sanford LP
	12/6/2010
	2
2
0
	H

	
	WD-40
	WD-40
	WD-40 Company
	6/8/2012
	1
4
0
	N/A

	
	Fellowes Shredder Lubricating Oil
	Lubricating oil
	Fellowes
	11/8/2010
	0
1
0
	N/A

	
	HP LaserJet Printer Cartridge CE285A
	Printer toner
	Hewlett-Packard Company
	10/18/2013
	1
1
0
	N/A


Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE
HP LaserJet Printer Cartridge C4127X
Printer toner
Hewlett-Packard Company

7/8/2010
1
1
0
N/A Purell Instant Hand Sanitizer
Hand sanitizer

GOJO Industries Inc.
4/29/2013
2
3
0
N/A Germ-X Hand Sanitizer
Hand sanitizer



Vi-Jon Inc.
6/10/2008
2
3
0
N/A Pure Citrus Orange Air Freshener
Air freshener


Blue Magic Inc.

1/1/2011
1
2
0

A Glade Country Garden Air Freshener
Air freshener
S.C. Johnson & Son Inc.

1/6/2005
0
4
0
N/A Crayons
Crayons
Greenbrier International
7/14/2007
1
1
0
N/A
Static Guard
Static guard
Alberto-Culver Company
10/19/1998
1
4
1
SR
Energizer Batteries
Batteries
Energizer Holdings Inc.
1/1/2013
N/A N/A N/A  N/A

Super Glue
Super glue

Pacer Technology
 1/30/2012
1
2
1
N/A M&R Marking Systems,
Ideal Stamp Ink Refill
Refill ink

LLC
1/24/1996   N/A N/A N/A  N/A

Goo Gone
Degreaser
Magic American Products  10/18/2010  1  2   0   N/A Canon GPR-23 Black Toner
Printer toner

Canon Inc.
1/21/2011 N/A N/A N/A N/A Canon GPR-23 Yellow Toner
Printer toner

Canon Inc.
1/21/2011   N/A N/A N/A  N/A

Utilities-Water Field

Canon GPR-23 Magenta Toner
Printer toner
Canon Inc.
1/21/2011   N/A N/A N/A  N/A

Trucks & Warehouse  Acetylene
Acetylene
Paxair
8/1/04
0
4
2 Calcium Hypochlorite
Accu-tab SI Calcium Hypochlorite Tablets


tablets PPG Industries
5/16/03
3
0
1

Alex Plus
Latex Caulk
DAP inc
11/19/03      1     1     0

Aluminum Oxide
Aluminum Oxide
DYNA Systems
11/19/03      0     0     0

Armor All
Armor All
Armor All
01/01/98      0     0     0

Acetone
Klean Strip Acetone

WM Barr
11/13/08 1 3 0 Brakleen Brake Cleaner
Brake Cleaner
CRC Industries
01/27/03 2 3  0 Cement Treated Base
CTB


Granite
9/21/02       0 0 0 Crafco Roadsaver
Modified Asphalt

Crafco inc
8/13/07       2     1     0

Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE
	Crack Filler
	Hot Applied crack filler
	Crack Filler corp.
	8/1/92
	1
	1
	0

	Cut Through Aerosol-Terpene
	Cut Through Aerosol
	Certified Labs
	8/29/07
	2
	4
	0

	Cutback Asphalt
	Cold Mix
	Granite
	8/21/02
	0
	0
	0

	Diesel w/ #2 Dye
	Diesel
	Chemtrec
	10/18/06
	1
	2
	0

	DD-135
	Sulonate mixture
	Chemsearch
	8/27/02
	2
	1
	0

	DUO Power-alkaline detergent
	Duo Power
	Chemsearch
	3/15/02
	2
	1
	0

	Durabond 70
	Epoxy
	Chemsearch
	9/8/04
	2
	1
	0

	Duraweld open arc
	welding wire
	X-Ergon
	8/9/01
	2
	0
	0

	Dylex PS
	Solvent
	Certified Labs
	4/27/00
	2
	4
	0

	Exalibur 7018 MR
	Covered Electrode
	Lincoln Electric Co.
	4/15/04
	1
	0
	0

	EZ Glide
	Aluminum Oxide
	Trust-X Partsmaster
	5/8/06
	1
	0
	0

	Fuel Additive
	Fuel Additive
	BG Products
	9/1/10
	2
	3
	0

	Flexible shrink tubing
	Polyolefins
	DYNA Systems
	4/11/01
	0
	0
	0

	Glue Redhot
	Redhot Glue
	Christys
	8/1/11
	2
	3
	1


Glass Cleaner

Green window glass
cleaner
Berkley Packaging
5/4/11
Gumout Starting Fluid
Starting Fluid
SOPUS
10/22/08
Grease
White Litium Grease
BG Products
4/26/10
GOJO hand cleaner
pumice hand cleaner
GOJO
4/27/11
Gasoline
Gasoline
Chemtrec
1/8/04
Graffiti-X
Graffiti-X
Champion Chemical
6/1/03 Grinding and cutting wheels
cutting wheels
United Abrasives inc
11/1/03 Hot Mix Asphalt
Hot Mix Asphalt

Granite
9/21/02
	Keynote
	Solvent
	Drummond American corp.
	5/1/01

	Klean Strip thinner
	paint thinner
	WM Barr
	12/14/05

	Lubricant Ease-on pipe joint
	Lubricant pipe joint
	Seacord corp
	2/1/06

	Maxi Lube ultra #2
	Maxi-Lube
	Chemsearch
	5/18/05

	Magnesium Oxide
	Ammonium sulfate/silica
	Chemsearch
	3/2/04

	Mistoil Aerosol
	Mistoil Aerosol
	Chemsearch
	12/30/04

	Naturalizer Aerosol
	Naturalizer Aerosol
	Chemsearch
	8/29/07


Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE

	MP 2-CYCLE TC-W3 MOTOR OIL
	2-CYCLE OIL
	Valvoline
	5/20/10
	1
	2
	n/a
	

	All Season Motor Oil 10W30
	10w30 Motor Oil
	Olympic Oil, LTD
	6/15/99
	1
	1
	0
	

	SAE 15W-40 Motor Oil
	15-40 Motor Oil
	Parts Plus
	5/7/10
	1
	1
	0
	

	2-Cycle Engine Oil
	2-CYCLE OIL
	Olympic Oil, LTD
	2/13/08
	1
	1
	0
	

	Oxygen Compressed
	Oxygen
	Praxair, Inc.
	9/1/04
	0
	0
	0
	

	Oatey Purple Primer/Cleaner
	Purple Primer
	Oatey
	8/2/12
	2
	3
	1
	

	Based Inverted Marking Paint
	Blue stake paint
	Rust-Oleum Corporation
	3/22/07
	2
	4
	0
	

	Marking Paint / Aerosols
	Blue stake paint
	Rust-Oleum Corporation
	11/16/12
	2
	4
	0
	

	PN-105 Aerosol
	
	Partsmaster, Div of NCH
	12/21/07
	2
	4
	0
	

	Premalube
	grease
	Premalube
	3/11/08
	1
	1
	0
	

	Marking Paint / Aerosols
	Blue-Blue stake paint
	Rust-Oleum Corporation
	11/16/12
	2
	4
	0
	

	Rustlick WS-5050
ITW Fluid Products Group
5/30/02
1
	1
	0
	

	Sealant
Great Stuff
Dow
1-37-08
1
	3
	0
	

	Solder
solder on terminals
Dyna Systems
7/6/99
1
	0
	0
	

	argon and carbon
Stargold
dioxideChemtrec
5/1/99
1
	0
	0
	

	Tank Tonic
Tank Tonic
Certified Labs
12/16/93
1
	1
	1
	

	Tekusolv aerosol
Tekusolv aerosol
Chemtrec Labs
4/14/08
1
	4
	0
	

	Thermo Gloss-Urethane
Thermo Gloss
Communication stds
4/20/94
1
	0
	0
	

	Thermo Strip floor finish
floor finish
Rochester midland
10/18/94
3
	0
	1
	

	Ultra Solv 221
solvent
Chemsearch
7/22/05
1
	1
	0
	

	WD40
Lubricant WD40
WD40
9/1/07
1
	2
	1
	

	Weld-on
pipe thread sealant
IPS corp.
12/1/11
1
	1
	0
	

	Yield Aerosol
Yield Aerosol
Chemsearch
12/21/07
2
	4
	0
	

	Z-disk
Aluminum oxide
Trust-X Partsmaster
5/12/04
1
	0
	0
	

	Glade Aerosol Spray
July 23
1
Aerosol Spray
S.C. Johnson & Son Inc.
2003
	4
	0
	


Feb. 24
2003
June 25
20010
Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer
D-Con Ready Mixed Baitbits
D-Con
Reckitt Benckiser Inc. FMG
UV Grease

Jet-Lube Inc.
Ortho Home Defense


Date
H
F
R
PPE
June 11
2009
2
0
0
B

Sep. 14
2009
0
1
0
N/A

Mar. 03
Ortho Insect Killer

Max
The Scotts Company Armor All Products

2011
1
0
0
Armor All Original Protectant
Armor All

Company
Jan. 1998
July 30

OFF! Deep Woods
OFF!
S.C. Johnson & Son Inc.

DAP Silicone
Silicone
DAP Products Inc.

Mobil SCH 630
Mobil Oil
Exxon Mobil Corporation
Parts Plus HD SAE 30
Parts Plus Motor Oil
Ashland
Speedaire PD Blower Oil
Blower Oil
Speedaire

2009
2
3
0
July 16

2013
1
1
0
Jan. 13
2009
0
1
0
Sep. 09
2010
1
1
0
Simple Green All-Purpose Cleaner
Simple Green
Sunshine Makers Inc.
Feb. 2012
1
0
0
0

Simple Green Degreaser
Simple Green
Sunshine Makers Inc.
Mar. 2011
1
0
0
0

Apr. 11

Mobil 600 XP 150
Mobil Gear Oil
Exxon Mobil Corporation
CRC Contact Cleaner
Contact Cleaner
CRC Industries Inc. WD-40 Water Resistant Silicone
WD-40

WD-40 Company

WD-40
WD-40
WD-40 Company

Permatex Ultra Blue RTV Silicone
RTV Silicone
ITW Permatex

Permatex Blue RTV Silicone
RTV Silicone
ITW Permatex


2011
0
1
0

July 05

2011
1
3
0

June 24
2011
June 08
2012
Feb. 23
2012
1
1
0

June 07
2012
1
1
0

Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE
Apr. 28


	Permatex AntiSeize
	AntiSeize
	Permatex Inc.
	2009
	1
	1
	1

	Christy's Purple Primer
	PVC Primer
	T Christy Enterprises Inc.
	Aug-11
	2
	3
	1

	Christy's Gray PVC Pipe Cement
	PVC Glue
	T Christy Enterprises Inc.
	Aug. 2011
	2
	3
	1

	Christy's Red Hit Blue Glue
	PVC Glue
	T Christy Enterprises Inc.
	Aug. 2011
	2
	3
	1

	Aervoe Marking Paint
	Marking Paint
	Aervoe Industries Inc.
	July 23
2003
	
	
	

	TFE Paste
	ACE Pipe Sealant
	William H. Harvey
Company
	Aug. 22
2002
	0
	0
	0

	ACE Rust Stop Emamel
	Paint
	ACE Hardwarw
Corporation
	Sep. 07
2007
	
	
	

	Strust
	Rust-Olueum Gray Paint
	Rust-Oleum Corporation
	May 43
2013
	2
	4
	0

	Green Fluorescent Paint
	Green Marking Paint
	Seymour of Sycamore
	Nov. 14
2013
	1
	3
	3

	Purple Temporary Marker
	Purple Marking Paint
	Seymour of Sycamore
	Nov. 14
2013
	1
	4
	3

	Tellus Oil 32
	Hydraulic Oil
	Equilon Enterprises
	Jan 04 1999
	1
	1
	0

	Maguire's G104
	Car Wash Soap
	MotorActive
	Feb. 2012
	0
	0
	0

	Waxie Green Clear Hand wash
	Soap
	Waxi Sanitary Supply
	Oct. 25
2011
	
	
	

	Purell SF 607
	Hand Sanitizing Foam
	GOJO Industries Inc.
	Jan. 12
2011
	
	
	

	Green Link Neutral Floor Cleaner
	Floor Cleaner
	ZEP
	Oct. 01
2012
	
	
	

	Streak Free Glass Cleaner
	Glass Cleaner
	Enforcer Products
	Aug. 13
2007
	
	
	

	All Purpose Cleaner and Degreaser
	Cleaner
	Zep
	Nov. 08
2007
	1
	0
	0

	Clorox Germicidal Bleach
	Bleach
	The Clorox Company
	Sep. 2012
	
	
	

	Greenclean Pro
	Algaecide
	BioSafe Systems L.L.C.
	
	
	
	


Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE
July 09


	70% Isopropyl Alcohol
	Rubbing Alcohol
	Cumberland-Swan Inc.
	2009
	

	Indicating Drierite
	Drierite
	W.A. Hammond Drierite
Company
	Jan. 03
1992
	

	Liquinox
	Liquinox
	Alconox Inc.
	Feb 24
2005
	

	Electrode Storage Solution
	Storage Solution
	AquaPhoenix Scientific Inc.
	Nov. 20
2009
	1
	0
	0

	Buffer Soln. PH 10.00
	PH Buffer 10.00
	AquaPhoenix Scientific Inc.
	Nov. 20
2009
	1
	0
	0

	Buffer soln. PH 7.00
	PH Buffer 7.00
	AquaPhoenix Scientific Inc.
	Nov. 19
2009
	1
	0
	0

	Buffer Soln. PH 4.00
	PH Buffer 4.00
	AquaPhoenix Scientific Inc.
	Nov 19
2009
	1
	0
	0

	Hydrogen Peroxide Solution
	Hydrogen Peroxide
	Aaron Industries
	Jan. 31
2012
	
	
	

	Sodium Hypochlorite
	Sodium Hypochlorite
	Hill Brothers Chemical Co.
	May 29
2013
	2
	0
	1

	Oxygen, Compressed
	Oxygen
	Praxair
	Dec. 2009
	0
	0
	0

	Acetylene, Dissolved
	Acetylene
	Praxair
	Dec. 2009
	0
	4
	2

	Liquid Paper Correction Tape
	White-out
	Liquid Paper
	Aug 1 2005
	
	
	

	Loctite® Threadlocker 2760 High
	Locktite
	Henkel
	Mar 20
2012
	
	
	

	Loctite Polyseamseal All Purpose Adhesive
Caulk
	Adhesive
	Henkel
	Feb 17
2011
	
	
	

	All Season T30 Select
	Air Compressor Oil
	Ingersol Rand
	Jan 01 2008
	
	
	

	Sulfuric Acid, 52-100%
	Sulfuric Acid
	Mallinckrodt Baker Inc.
	Feb. 04
2005
	3
	0
	2

	Sodium Thiosulfate 0.01 to 1.0 Normal
Volumetric Solutions
	Sodium Thiosulfate
	Mallinckrodt Baker Inc.
	July 29
2003
	1
	0
	0


Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE
	
	Sodium Sulfite
	Sodium Sulfite
	Mallinckrodt Baker Inc.
	Feb. 15
2008
	2
	0
	0
	

	
	
	
	
	Oct. 28
	
	
	
	

	
	Sodium Hydroxide Solutions
	Sodium Hydroxide
	Mallinckrodt Baker Inc.
	2004
	3
	0
	1
	

	
	
	
	
	Sep 09
	
	
	
	

	
	Potassium Citrate, Monobasic
	Potassium Citrate
	GFS Chemicals
	2004
	
	
	
	

	
	
	
	
	June 06
	
	
	
	

	
	Nitric Acid, 50-70%
	Nitric Acid
	Mallinckrodt Baker Inc.
	2005
	3
	0
	0
	

	
	
	
	
	Sep. 24
	
	
	
	

	
	Hydrochloric Acid, 33-40%
	Hydrochloric Acid
	Mallinckrodt Baker Inc.
	2004
	3
	0
	0
	

	
	
	
	
	Aug. 18
	
	
	
	

	
	Ascorbic Acid
	Ascorbic Acid
	Mallinckrodt Baker Inc.
	2005
	1
	0
	0
	

	
	
	
	
	Aug. 18
	
	
	
	

	
	Ammonium Chloride
	Ammonium Chloride
	Mallinckrodt Baker Inc.
	2005
	2
	0
	0
	

	
	
	
	
	Oct. 10
	
	
	
	

	
	Zinc Acetate 2N
	Zinc Acetate
InkJet Printer Toner
	Sciencelab.com Inc.
	2005
Nov. 20
	1
	0
	0
	A

	
	InkJet Printing Toner
	Cartridge
	Hewlett - Packard Co.
	2009
	1
	2
	0
	

	Community
	
	
	
	
	
	
	
	

	Development
	
	
	
	
	
	
	
	

	(Sanders' Shed)
	Resolve Carpet Cleaner
	Carpet cleaner
	Resolve
	
	
	
	
	

	
	Windex
	Multisurface cleaner
	Johnson
	
	
	
	
	

	
	Spectracide
	Bug stop Home Barrier
	Spectrum Group
	
	
	
	
	

	
	Hand Sanitizer
	Anti-bacterial Hand San
	Bath & Body Works
	
	
	
	
	

	
	Finest All Purpose Oil
	Cooler & Motor Oil
	Dial Manufac
	
	
	
	
	

	
	GermX
	hand sanitizer
	GermX
	
	
	
	
	

	
	Clorox Disinfecting Wipes
	disinfecting wipes
	Clorox
	
	
	
	
	

	
	Froggy's Flakes Snow Fluid
	artificial snow
	Froggy's Fog LLC
	
	
	
	
	

	
	Elmers Glue All
	craft glue
	Elmers
	
	1
	1
	0
	

	
	Equate Sport SPF50 Sunscreen
	sunscreen
	Fruit of the Earth Inc
	
	
	
	
	

	
	Aleene's Original Tacky Glue
	glue
	iLove to Create
	
	
	
	
	

	
	Apple Barrel Paints
	craft paint
	Plaid Enterprises
	
	
	
	
	



Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE

Court

Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE
CC533A
PRINTER CARTRIDGE
HP
3/16/2009
1
1
0

CIP

Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer
409 Degreaser
Degreaser
Clorox

Date
H
F
R
PPE

	Police-103/MMC
	Waxie Liquid Soap
	Liquid Soap
	Waxie

	
	Toilet Bowl Cleaner
	Toilet Bowl Cleaner
	

	
	Glass and Surface Cleaner
	Glass and Surface
Cleaner
	

	
	Shine Up
	Shine Up
	Diversy

	
	Purell Hand Sanitizer
	Purell
	GoJo Industries

	
	Crew Natural Bowl Cleaner
	Crew Natural Bowl
Cleaner
	Diversy

	
	Multi Surfaces
	Multi Surfaces
	

	
	Gum Remover
	Gum Remover
	

	
	Stride Floor Cleaner
	Stride Floor Cleaner
	Diversy

	
	Printer Cartridge
	92298X
	Hewlett Packard

	
	Printer Cartridge
	CE5054X
	Hewlett Packard

	
	Printer Cartridge
	C7A15X
	Hewlett Packard

	
	Printer Cartridge
	CE278A
	Hewlett Packard

	
	Printer Cartridge
	Cannon GPR 10
	Cannon

	
	Printer Cartridge
	Cannon GPR 15
	Cannon

	
	Printer Cartridge
	HP 14
	Hewlett Packard

	
	Printer Cartridge
	HP 56
	Hewlett Packard

	
	Printer Cartridge
	HP 58
	Hewlett Packard

	
	Toner Cartridge GPR-22
	DAJ03795
	Cannon

	
	Correction Fluid
	Correction Fluid
	Office Max

	
	Liquid Paper Correction Pen
	Correction Pen
	Papermate

	
	Red Refill Ink
	Red Refill Ink
	Office Max

	
	Blue Refill Ink
	Blue Refill Ink
	Office Max

	
	Red Refill Ink
	Red Refill Ink
	Cosco

	
	Oust Air Sanitizer Surface Disinfectant
	Surface Disinfectant
	Johnson & Johnson

	
	409 All Purpose Cleaner
	409 All Purpose Cleaner
	Clorox

	
	Professional Resolve Fabric Cleaner
	Resolve
	Reckitt & Coleman

	
	Hydrogen Peroxide
	Hydrogen Peroxide
	Walgreen's


Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer
Disinfectant Wipes
Disinfectant Wipes
Clorox
Lubriderm
Lubriderm
Johnson & Johnson
Toner Cartridge GPR-22
DAJ03795
Cannon
Printer Cartridge
C4811
Hewlett Packard

Printer Cartridge
C4810
Hewlett Packard

Printer Cartridge
C4812
Hewlett Packard

Printer Cartridge
C4813
Hewlett Packard

Printer Cartridge
C8061X
Hewlett Packard

Plotter Design Jet 500
HP82, HP11, HP10
Hewlett Packard

Printer Cartridge
CE278A
Hewlett Packard

Windex
Windex
SC Johnson

Formula 409 Cleaner
409
The Clorox Company

Dawn Dish Soap
Dawn
Procter & Gamble

GoJo Hand Cleaner
GoJo Hand Cleaner
GoJo Industries
GermX
GermX
ViJon
Butane
Butane
Kleen Strip
Acetone
Acetone
Ronson

Deglazer Cleaner
Deglazer
Fliebing's
Lysol Disinfectant Wipes
Lysol Disinfectant Wipes
Rickett Industries
Gorilla Super Glue
Super Glue
Gorilla Glue Co.
Green Works Cleaner
Green Works
Clorox
Rust-Oleum Marking Paint
Rust-Oleum
Rust-Oleum
Printer Cartridge
CX410DE
Lexmark

Printer Cartridge
CE410A
Hewlett Packard

Printer Cartridge
CE278A
Hewlett Packard

Plotter Design Jet 500
HP82, HP11, HP10
Hewlett Packard

GermX
GermX
ViJon
Isopropyl Rubbing Alcohol
Rubbing Alcohol
CVS
Printer Cartridge
64035HA
Lexmark

Palmolive Dish Soap
Dish Soap
Colgate Palmolive

Date
H
F
R
PPE
Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE
409 All Purpose Cleaner
409 All Purpose Cleaner
Clorox Disinfecting Office

3M Disinfecting Office Cleaner

Cleaner
3M
Lysol Disinfectant Spray
Lysol Disinfectant Spray
Rickett Industries
Safetec Hand Sanitizer
Hand Sanitizer
Safetec of America Office Depot Cleaning
Office Depot Cleaning Duster


Duster
Office Depot
Ross Envelope Stik
Ross Envelope Stik
Ross

Printer Cartridge
Q2613A
Hewlett Packard

Printer Cartridge
C7115A
Hewlett Packard

Toner Cartridge
DAJ03795
Cannon
Expo2 White Board Cleaner
Expo2
Sanford

Sterile Isotonic Buffered Flush
Sterile Flush
Cintas
Isopropyl Alcohol
Isopropyl Alcohol
Cintas
Burn Relief Gel Pack
Burn Relief
Rye Pharmaceuticals
Triple Antibiotic Ointment
Antibiotic Ointment
Hart Health
Antiseptic Towelett
Antiseptic Towelette
Nice-Pak Products
Fast Burn Relief
Fast Burn Relief
Affirmed Medical
Hydrogen Peroxide
Hydrogen Peroxide
Cintas
Burn Spray
Burn Spray
Cintas Kroger Disinfectant
Kroger Disinfectant Wipes

Wipes
Kroger

Lysol Disinfectant Spray
Lysol Disinfectant Spray
Rickett Industries
Printer Cartridge
501A
Hewlett Packard

Printer Cartridge
15A
Hewlett Packard

Printer Cartridge
C7115A
Hewlett Packard

Printer Cartridge
CE278A
Hewlett Packard

Toner Cartridge
DAJ03795

Cannon Police-107/MOC Ina
Printer Cartridge
CE505X
Office Depot
Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE
	Printer Cartridge
	Q2613X
	Hewlett Packard

	Printer Cartridge
	CF280A
	Hewlett Packard

	Printer Cartridge
	CE278A
	Hewlett Packard

	Pledge
	Pledge
	Johnson & Johnson

	Lysol Disinfectant Spray
	Lysol Disinfectant Spray
	Rickett
	
	
	
	

	Liquid Paper
	Liquid Paper
	Newwell-Rubbermaid Inc.
	
	
	
	

	Purcell Advanced Refreshing Gel
	Refreshing Gel
	GoJo Industries
	
	
	
	

	Great-Value Disinfecting Wipes
	Disinfecting Wipes
	Wal-Mart
	
	
	
	

	P.A.W.S. Hand Wipes
	Hand Wipes
	Safetec of America
	
	
	
	

	Cannon GPR 22 Toner
	Toner
	Cannon
	
	
	
	

	BugX Towelette
	Insect Repellant
	Coretex Products
	
	
	
	

	Rust-Oleum Marking Paint
	Marking Paint
	Rust-Oleum
	
	
	
	

	Pennzoil Brake Fluid (12 oz)
	Brake Fluid
	Pennzoil Products
	
	
	
	

	Valvoline Automatic Transmission Fluid
	Transmission Fluid
	Mercon V
	
	
	
	

	Valvoline 20W-50
	Motor Oil
	Mercon V
	
	
	
	

	Carquest 10W-40
	Motor Oil
	Ashland
	
	
	
	

	Mobil 1 Racing 4T 10W-40
	Motor Oil
	Mobil
	
	
	
	

	Carquest Power Steering Fluid
	Power Steering Fluid
	Ashland
	
	
	
	

	Goo Gone
	Goo Gone
	Magic America Products
	
	
	
	

	WD 40
	WD 40
	WD 40 Co
	6/8/12
	1
	4
	0

	Sudecon Irritant Spray Decontamination Wipe
	Sudecon
	Fox Labs International
	

	NarcoPouch 901
	NarcoPouch
	ODV Inc.
	

	NarcoPouch 902
	NarcoPouch
	ODV Inc.
	

	NarcoPouch 904B
	NarcoPouch
	ODV Inc.
	

	NarcoPouch 905
	NarcoPouch
	ODV Inc.
	

	NarcoPouch 909
	NarcoPouch
	ODV Inc.
	

	NarcoPouch 923
	NarcoPouch
	ODV Inc.
	

	NarcoPouch 924
	NarcoPouch
	ODV Inc.
	

	Mark II Cocaine Wipe
	Cocaine Wipe
	Sirchie
	

	Mark II Mecke's Reagent
	Mecke's Reagent
	Sirchie
	



Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)

Chemical/Product Name

Common Name

 Manufacturer  Purell Hand Sanitizer
Purell

GoJo Industries McKesson Alcohol Prep Pad
Alcohol Prep Pad


McKesson Awesome Bleach
Bleach
Awesome Products
Formula 409 Cleaner
409
The Clorox Company

Printer Cartridge
CF280A
Hewlett Packard

Toner Cartridge GPR-22
DAJ03795
Cannon Clorox Disinfectant

Date
H
F
R
PPE
Disinfectant Wipes

Wipes
Clorox
Bleach Spray
Clorox Clean-Up
Clorox
Resolve Fabric Refresher
Resolve

Rickett & Coleman Office Depot Cleaning Duster Spray
Cleaning Duster Spray


Office Depot Kleen Air Odor Control
Kleen Air Odor Control
Waxie Sanitary Supply
Pledge Furniture Spray
Pledge Furniture Spray
Johnson & Johnson
Lysol Disinfectant Spray
Lysol Disinfectant Spray

Rickett Printer Cartridge Room 2A136
BC05X
Office Depot
Printer Cartridge Room 2A135
CE278A
Hewlett Packard

Printer Cartridge Room 2A133
Q613X
Hewlett Packard

Printer Cartridge Room 2A131
CE278A
Hewlett Packard Triple Antibiotic Ointment
Antibiotic Ointment

Hart Health
Antiseptic Towelett
Antiseptic Towelette
Nice-Pak Products
Burn Relief Gel Pack
Burn Relief
Rye Pharmaceuticals Benzalkonium Chloride Towelette
Antiseptic Towelette

Triad Disposables Expect Antiseptic Spray
Antiseptic Spray


Cintas
Hydrogen Peroxide
Hydrogen Peroxide
Cintas
Ocufresh Eye Wash
Eye Wash

Cintas Technology Services  Ajax Quick-solv
Spray Cleaner
Colgate - Palmolive
Misty Multipurpose Cleaner
Spray Cleaner
Amdep

Tech Spray Wipes
Alcohol Wipes
TechSpray

Toner
Printer Toner
Brother
Toner
Printer Toner
Hewlett Packard

Inkjet Ink
Printer Ink
Brother
Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer
Inkjet Ink
Printer Ink
Hewlett Packard

Simple Green Cleaner
Simple Green
Sunshine Makers

Artic Silver
Terminal Grease
Artic Silver
Toner
Copier toner
Canon
Cleaner Duster
Canned Air
Office Depot Finance
Lysol wipes


Lysol
Toner
Brother/Canon
Liquid correction fluid
Office depot
Air duster
Falcon Safety

Hand sanitizer
Purell
Refill ink
Ideal stamps
2000 Plus
Permanent marker
Sharpie
Screen cleaning wipes
Ativa
Human Resources
Toner
toner
Brother/HP Marks-A-Lot permanent marker
permanent marker

Avery Sharpie Magnum permanent marker
permanent marker

Sharpie Expo Click low odor dry erase
dry erase marker

Expo

Scotch photo mount
photo mount
Scotch

3M desk & office cleaner wipes
cleaner wipes
3M
3M disinfecting office cleaner
office cleaner
3M
Static Guard
static guard
static guard
Expo white board care
white board cleaner
Expo

Cleaning duster
cleaning duster
Office Depot
Sharpie fine point marker
permanent marker
Sharpie
Red refill ink
refill ink
Office Depot
Sharpie accent highlighter
highlighter
Sharpie
Energizer AA battery
battery
Energizer
Elmer's washable school glue stick
glue stick
Elmer's


Date
H
F
R
PPE
Legal


Office Depot Cleaning Duster
Cleaning Duster

Glycol Ink/PVC
Stamp Pad refill ink

Falcon Safety Products,
Inc.
10/28/2009
1
4
1
Microcell Products
Company
Not dated
Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE
Clorox Disinfecting Wipes
Disinfecting Wipes
Clorox
5/11/2011
Elmer's Washable School Glue Stick
Glue Stick

Elmer's Products, Inc.
5/11/2011
1
0
0 Sharpie Permanent Marker
Marker
Newell Rubbermaid, Inc.

5/4/2010
Good Rich Technologies
Office Depot Correction Tape
Correction Tape


Co., Ltd.
11/10/2007

Vanilla Bean Anti-Bacterial Hand Gel
Anti-Bacterial Hand Gel
Tri Tech Laboratories, Inc.  10/11/1997
Glade Aerosol Ferns & Blossoms Spray
Deodorizing Spray
S.C. Johnson & Son, Inc.
1/23/2006
1
4
0 Kiwi Black Leather Premium Shine
Shoe Polish

Kiwi Brands Inc.
12/4/1990
	Easy Gardener Products,
Jobes Fertilizer Spikes
Plant fertilizer
Inc.
11/27/2007
1
SuperThrive
Plant food
SuperThrive
None
	0
	0
	

	Pledge Furniture Spray - Lemon Fresh
Furniture spray
S.C. Johnson & Son, Inc.
1/17/2012
1
	3
	0
	

	Kiwi Express Shine Instant Shoe Sponge
Shoe Polish
Kiwi Brands Inc.
3/17/2005
1
	2
	0
	

	HP LaserJet C4127A-D-X-XC Print Cartridge  Toner
Hewlett-Packard Company
7/8/2010
1 TN-450 Black Toner Cartridge
Toner

Brother Industries, Ltd.
6/10/2010
	1
	0
	

	HP LaserJet CE255A-X-XC-XD Print
Cartridge
Toner
Hewlett-Packard Company
6/4/2012
1
	1
	0
	

	HP LaserJet CE278A-AC-AD-AF Print
Cartridge
Toner
Hewlett-Packard Company  10/18/2013
1
	1
	0
	

	HP LaserJet Q6000A-AD Black Print
Cartridge
Toner
Hewlett-Packard Company
5/3/2011
1
	1
	0
	

	HP LaserJet Q6001A Cyan Print Cartridge
Toner
Hewlett-Packard Company
3/11/2009
1
	1
	0
	

	HP LaserJet Q6002A Yellow Print Cartridge
Toner
Hewlett-Packard Company
3/11/2009
1
	1
	0
	

	HP LaserJet Q6003A Magenta Print Cartridge Toner
Hewlett-Packard Company
4/13/2012
1
	1
	0
	

	Airport
	Degreaser
	Degreaser
	Mantek
	
	
	
	
	

	
	Brakleen Brake Parts Cleaner
	Brake Parts Cleaner
	CRC
	11/28/2012
	2
	0
	0
	B

	
	Motor Oil
	Parts Plus Motor Oil
	Parts Plus
	9/9/2010
	1
	1
	0
	

	
	Air Cooled 2-Cycle Engine Oil
	2-CYCLE Oil
	Citgo Petroleum
	11/9/2012
	1
	1
	0
	

	
	Bar & Chain Oil
	Bar Oil
	Stihl Inc.
	1/2/2009
	1
	1
	0
	


Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE
	
	Diesel Fuel
	Diesel
	TW Brown LLC
	1/31/1999
	1
	2
	0

	
	Gasoline
	Gasoline
	Petrocom Energy, LLC
	3/5/2008
	1
	3
	0

	
	Mystik Grease
	Grease
	Citgo Petroleum
	6/20/2011
	1
	1
	0

	
	Epoxy Resin
	Epoxy
	West Systems Inc.
	4/26/2013
	1
	2
	0

	
	Concrete Sealant
	Concrete Sealant
	Enviroseal Corp.
	5/4/2010
	1
	0
	0

	
	Latex White Paint
	Latex White Paint
	General Paint Corp.
	7/3/2012
	1
	0
	0

	
	BG Glass Cleaner
	Glass Cleaner
	BG Products Inc.
	1/25/2012
	3
	1
	0

	Planning
	Lysol Clean Wipes
	Sanitizing Wipes
	Lysol
	
	
	
	

	
	White Board Cleaner
	white board cleaner
	Sanford
	
	
	
	

	
	White Out
	Correction Fluid
	Office Depot
	
	
	
	

	
	Toner
	Toner
	Cannon
	
	
	
	

	
	Antibacterial Gel
	Hand Sanitizer
	Bath 7 Body
	
	
	
	

	
	Static Guard
	static guard
	B & G Foods
	
	
	
	

	
	Keyboard Cleaner
	Cleaning Duster
	Office Depot
	
	
	
	

	
	Surface Cleaner Dry Erase Markers
	Desk Office Cleaner Dry Erase Markers
	3M
Sanford
	
	
	
	

	Town Clerk
	Cleaning Duster
	Compressed Air
	Office Depot
	
	
	
	

	
	Disinfecting Wipes
	Cleaning Wipes
	The Clorox Company
	
	
	
	

	
	Permanent marker
	Sharpie
	Sharpie
	
	
	
	

	
	White out
	Correction Tape
	Office Depot
	
	
	
	

	
	White Board Cleaner Stamp Pad Ink
	White Board Cleaner Liquid Ink
	Expo, Inc. Office Depot
	
	
	
	

	Town Manager
	Murphy's Oil Soap
	Wood Cleaner
	Colgate
	
	
	
	

	
	Old English
	Scratch Remover
	Reckett Benckiser
	
	
	
	

	
	Great Value Vinegar
	Vinegar
	Mizkan (Walmart Generic)
	9/13/2012
	1
	0
	0

	
	Comet-Disinfectant Powder Cleanser
	Cleaner
	Prestige Brands
	
	
	
	

	
	Satin Furniture Polish
	Furniture Polish
	Brady
	
	
	
	

	
	Red Max Stainless Steel Cleaner
	Cleaner
	Sales Source, Inc.
	
	
	
	

	
	Sharpie
	permanent marker
	ACMI
	
	
	
	

	
	Highlighter
	Marker
	ACMI
	
	
	
	

	
	Desk Wipes
	Wet Cleaners
	3M
	
	
	
	

	
	Dust Destroyer
	Air in a can
	Falcon Safety Products
	
	
	
	



Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE
	
	Dry Erase Board Cleaner
	Cleaner
	ACCO-Bount
	

	
	Spray Adhesive
	Glue
	3M
	

	
	White Out
	Cover-Up
	Sanford
	

	
	Glue Stick
	Glue
	Seventh Avenue Trending
	

	
	Dish Soap
	Soap
	Procter & Gamble
	

	
	Colorbrust Plant Food
	Plant Food
	United Industries Corp
	

	
	
	Computer Screen
	
	

	
	Cleaning Wipes
	Cleaner
	Beklin
	

	
	Hand sanitizer Windex
	Sanitizer Windex
	Safeway Generic Johnson & Johnson
	

	Public Works
	Baking Soda
	Baking Soda
	Church & Dwight
	11/1/10
	1
	0
	0
	E

	
	Bleach
	Clorox Bleach
	Clorox
	7/12/11
	3
	0
	1
	E

	
	Comet-Disinfectant Powder Cleanser
	All-Purpose Cleanser
	Procter & Gamble
	11/1/11
	1
	0
	0
	B

	
	Lava Soap
	Lave Soap
	WD-40 Company
	12/4/10
	1
	0
	0
	N/A

	
	Light Acid Bowl Cleaner
	Toilet Bowl Cleaner
	Canberra Corp.
	3/24/10
	3
	0
	0
	B

	
	Pine Sol Cleaner
	Cleaner
	Clorox Sales Company
	6/5/12
	2
	0
	1
	B

	
	Quality Care Furniture Polish Lemon Scent
	Furniture Polish Lemon
	Cleaning Solutions Group
	9/15/11
	2
	2
	0
	B

	
	Raid Flying Insect Killer
	Raid Bug Killer
	SC Johnson
	5/4/11
	1
	4
	0
	B

	
	Simple Green All-Purpose Cleaner
	Cleaner
	Sunshine Makers
	1/9/13
	0
	0
	0
	B

	
	Windex Glass Cleaner
	Windex
	Drackett Professionals
	4/8/10
	0
	1
	0
	B

	
	Diesel 2 Dyed
	Diesel Fuel 2 Dyed
	Canyon State/Shell
	4/29/12
	1
	3
	0
	B

	
	Gasoline
	Gasoline
	Canyon State/Motiva
	1/18/12
	1
	3
	0
	B

	
	500 All Purpose Gear Oil 80W-90
	Gear Oil
	Canyon State
	3/25/12
	0
	1
	0
	B

	
	Quaker State Formula Ultra 5W-30
	Motor Oil
	Canyon State
	12/11/11
	0
	1
	0
	B

	
	Zerex Original Antifreeze Coolant ZX002
	Antifreeze/Coolant
	Ashland
	8/29/10
	1
	1
	0
	B

	
	Outershield 70 & 70-H
	Flux Cord Electrode
	Lincoln Electric Co.
	10/1/11
	2
	0
	0
	A

	
	Acetylene
	Acetylene
	Airgas
	5/11/11
	1
	4
	3
	B

	
	Argon
	Argon
	Airgas
	5/6/13
	0
	0
	0
	C

	
	Carbon Dioxide
	Carbon Dioxide (CO2)
	Airgas
	1/20/12
	1
	0
	0
	B

	
	Oxygen
	Oxygen (02)
	Airgas
	9/24/13
	0
	0
	0
	OX

	
	Nitrogen
	Nitrogen
	Airgas
	9/27/12
	0
	0
	0
	X

	
	Propane
	Propane
	Airgas
	8/19/13
	2
	4
	0
	B



Department

Hazard Communication Chemical Inventory
Town of Marana Inventoried as of December 9, 2013
Town Wide (Organized by Department)


SDS Rev.


NFPA 704 Label
(Storage Location)
Chemical/Product Name
Common Name
Manufacturer

Date
H
F
R
PPE
	Yellow77 Wire Pulling Lubricant
	Wire Pulling Lub
	Ideal Industries, Inc.
	3/10/2006
	2
	0
	0

	WD 40 Aerosol
	WD 40
	WD 40 Company
	6/8/2012
	3
	1
	0

	Misty Wasp & Hornet Spray
	Wasp Spray
	Amrep Inc.
	1/27/2008
	1
	2
	0

	Thompson Water Seal
	Thompson Water Seal
	Thompson & Formby Inc.
	4/11/2011
	1
	1
	0

	Tide Liquid Soap
	Tide Soap
	Procter & Gamble
	3/5/2013
	2
	1
	0

	Surlfan AS Herbicide
	Surflan
	Southern Agricultural
	2/1/1998
	2
	0
	0

	Klean Stripper
	Klean Strip Acetone
	WM Barr
	3/28/2012
	2
	1
	0

	Scotchkote Brand Electrical Coating
	Scotchkote
	3M
	7/21/2011
	1
	0
	0

	Scotchcast Resin 4 Kit
	Scotchcast Resin
	3M
	10/17/2011
	2
	1
	0

	Spray Gun & Parts Cleaner
	Spray Gun Cleaner
	Supreme Chemicals of
Georgia
	2/1/2011
	1
	0
	0

	Sea Foam
	Sea Foam
	Sea Foam Sales Company
	1/1/2013
1
3
0

	Orange Spray Pain Rust-Oleum
	Orange Spray Paint
	Rust-Oleum
	9/28/2011
2
4
0

	Stripe Safety Red
	Marking Paint
	Seymour of Sycamore
	1/14/2013
1
3
3

	Round-Up Original MAX Herbicide
	Round-Up
	Monsanto Company
	10/28/2006
1
1
0

	QPR VOC Repair Material
	Cold Patch
	QPR Pavement Repair
	5/1/2012
1
1
0

	Propane
	Propane
	International Industrial
Gases, LTD
	10/8/2013
1
4
0

	Pledge Furniture Polish-Lemon Cleaner
	Pledge
	Johnson & Johnson
	6/10/2011
0
4
0

	Pine Sol Cleaner
	Pine Sol
	Clorox Company
	10/21/2011
1
1
0

	Oatey Purple Primer
	PVC Primer
	Oatey Co.
	5/20/2005
2
5
1

	PVC Plastic Pipe Cement
	PVC Glue
	Carlon Electrical Products
	9/22/2005
2
3
1

	Piston Lube/Separating Oil
	Piston Lube
	DK Rosedale LLC
	8/9/2009
1
1
0

	PB Penetrating Catalyst
	PB Blaster
	The Blaster Company
	11/15/2010
2
2
0

	Ultra Palmolive Soap Original
	Palmolive Soap
	Colgate-Palmolive Co.
	3/30/2010
0
1
0

	Quali-Pro Oryzalin
	Oryzalin
	Farmsaver.com, LLC
	9/24/2008
2
0
0

	Mystik lubricants JT-6 Grease #2
	Mystik Grease
	Citgo Petroleum Corp.
	6/20/2011
1
1
0

	Max Force Fine Insect Bait
	Maxforce
	Bayer Environmental
Science
	1/27/2003
1
2
0

	Lysol Disinfectant Spray
	Lysol
	Reckitt Benckiser
	2
3
0

	Li 700 With Leci-Tech Surfactant
	LI 700
	Loveland Products Inc.
	9/11/2013
2
1
0




	LN-901 Heavy Duty AHE 90124TNO
	Liquid Nails
	Akzo Nobel Paints, LLC
	12/3/2013
	2
	3
	0

	Latex Caulk
	Latex Caulk
	DAP Inc.
	3/16/2009
	1
	0
	0

	Unleaded Gasoline
	Gasoline
	Petrocom Energy, LLC
	3/5/2008
	1
	3
	0

	
	Ultra Dawn Soap
	
	
	
	
	

	Ultra Dawn Soap
	Antibacterial
	Procter & Gamble
	12/20/2011
	1
	1
	0

	Cat Extended Life Coolant
	Cat Anti-Freeze
	Sopus Product
	10/30/2013
	2
	1
	0

	Clorox Disinfecting Wipes
	Clorox Wipes
	Clorox Company
	
	0
	0
	0

	Cleaner/Penetrant
	Cleaner/Penetrant
	Crown
	
	1
	1
	1

	Clear Caulk
	Clear Caulk
	Red Devil, Inc.
	Feb-96
	1
	1
	0

	Clear Wood Finish Gloss
	Clear Wood Finish
	Deft, Inc.
	8/24/2010
	2
	3
	0

	2 Cycle Engine Oil
	2 Cycle Engine Oil
	Citgo Petroleum
	1/25/2010
	1
	0
	0

	CLR Calcium Lime & Rust Remover
	CLR
	Jelmar
	11/1/2004
	1
	0
	0

	PolyFlex Type 3
	Crack Sealant
	Crafco Inc.
	6/1/2012
	
	
	

	Clean-n-Dip
	Clean-n Dip
	Dynacraft Industries, Inc.
	1/1/2006
	1
	1
	0

	Bug x 30 Insect Repellent
	Bug X
	Core Tex Products, Inc.
	11/16/2006
	1
	1
	0

	Brake Parts Cleaner
	Brake Cleaner
	CRC Industries
	1/15/2009
	2
	0
	0

	Battery Clean-Up Battery Clean
	Battery Clean
	The Noco Company
	3/15/2011
	1
	4
	0

	Air & Fabric Odor Eliminator
	Febreze
	Zep Commercial Sales
	3/2/2009
	1
	1
	0

	Anti-Seize Lubricant
	Anti-Seize
	Permatex, Inc.
	3/2/2009
	2
	4
	1

	Aquabac Biological Larvicide
	Aqua Bac
	AFA Environment, Inc.
	10/14/2013
	0
	0
	0

	Pure Citrus Air Freshener-Spray Armor All Multi-Purpose Cleaner
	Citrus Air Freshener Armor All
	Blue Magic Inc.
STP/Products Company
	5/22/2008
	1
	2
	0


REVISION HISTORY
	REV
	DESCRIPTION OF CHANGE
	DATE

	OR
	Original Release
	12/17/13

	REV
	Updated by Safety Office to include GHS Pictograms, included attachments, Master Chemical Inventory and annual review (Revision History).
	09/01/14

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	


Caution: A copy of this Safety Directive is an uncontrolled document. It is your responsibility to ensure you are using the current version. The electronic version is the only acceptable and controlled Safety Directive.

�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�









0�



0�



0�
�
�
0�
2�
0�
�
�
1�
3�
0�
�
�
1�
1�
0�
�
�
1�
3�
0�
�
�
2�
2�
0�
�
�
1�
0�
0�
�
�
0�
0�
0�
�
�
2�
0�
0�
�
�
1�
2�
0�
�
�
0�
0�
0�
�
�
0�
0�
0�
�
�
1�
0�
0�
�
�
1�
4�
0�
�
�
2�
4�
0�
�
�






Utilities-Wastewater�
�
�
�
�



ActiClean Gel	ActiClean�



Jacklyn Industries�
�
�



Clorox Toilet Cleaner	Toilet Cleaner�



The Clorox Company�
�






Coppertone Sunscreen Spray�
sunscreen�
Schering-Plough�
�
�
Expo dry erase markers�
dry erase markers�
Sanford LP�
�
�
Sharpie Fine Point Markers�
sharpies�
Sanford LP�
�
�
Amor All Ultra Shine Wash & Wax�
Amor All�
�
�
�
Dial Cooler Cleaner�
cleaner�
�
�
�
Great Stuff Insulating Foam gap & crack�
�
�
�
�
sealer�
insulating foam�
�
�
�
Zep Commercial glass cleaner professional�
�
�
�
�
strength�
glass cleaner�
�
�
�
Round Up Concentrate Plus Weed Killer�
Round up�
�
�
�
Weed Stopper II Pre-emergence herbicide�
pre-emergent�
�
�
�
Paint�
paint�
�
�
�
WD 40�
lubricant�
�
�
�
Aervoe Survey marking paint�
marking paint�
�
�
�
Oatey Abs to PVC transition cement�
glue�
�
�
�
Weld on 773 ABS�
weld�
�
�
�
Rust O Leum marking paint�
spray paint�
�
�
�
Dap Concrete & Mortar filler/sealer�
sealer�
�
�
�
Home Armor premium flexible sealer�
sealer�
�
�
�
Tile Lab Grout & Tile Sealer�
grout�
�
�
�
Dap All Purpose Painter acrylic Latex caulk�
caulk�
�
�
�
Gasoline


Minwax wood finish pen/stain/seal�
gas pen�
�
�
�
PLEDGE FURNITURE POLISH�
DUSTING SPRAY�
SC JOHNSON�
9/7/2011�
1�
3�
0�
�
LIQUID CORRECTION FLUID/TAPE�
WHITE OUT�
BIC/OFFICE DEPOT�
8/1/2005�
�
�
�
�
DISINFECTING/CLOROX WIPES�
CLOROX WIPES�
CLOROX/LYSOL�
�
�
�
�
�
ALCOHOL�
HAND SANITIZER /GEL�
PURELL�
11/15/2004�
1�
3�
0�
�
BALL PEN INK�
BALL PEN INK�
VARIOUS�
6/10/2005�
1�
1�
0�
�
HPCC364A�
PRINTER CARTRIDGE�
HP�
�
�
�
�
�
CANON E20�
PRINTER CARTRIDGE�
CANON�
1/17/2008�
�
�
�
�
HP 36A�
PRINTER CARTRIDGE�
HP�
5/24/2007�
1�
1�
0�
�
HP12A�
PRINTER CARTRIDGE�
HP�
2/15/2006�
1�
1�
0�
�









ALCOHOL�
HAND SANITIZER


/FOAM�



PURELL�



11/19/2009�
2	3	0�
�
Bleach�
Clorox Bleach�
Clorox�
�
�
�
Batteries (assorted)�
Battery�
Energizer Holdings�
�
�
�
Buffer Powder Pillows�
Buffer Powder�
Hach�
0/0/2012�
2�
1�
0�
n/a�
�
Buffer Solution for Detergents�
Buffer Solution�
Hach�
0/0/2009�
2�
0�
0�
n/a�
�
Buffer Solution Hardness�
Solution Hardness�
Hach�
0/0/2012�
2�
1�
0�
n/a�
�
Cleaning Duster�
Dust Spray�
Swinton Avenue Trading�
�
�
�
�
�
�
Clorox Clean Up Spray�
Clorox Clean Up Spray�
Clorox�
�
�
�
�
�
�
Comet-Disinfectant Powder Cleanser�
All-Purpose Cleanser�
Colgate Palmolive�
�
�
�
�
�
�
Demineralizer�
Demineralizer�
Hach�
0/0/2013�
1�
1�
0�
n/a�
�
Detergents Reagent�
Detergents Reagents�
Hach�
0/0/2012�
3�
0�
0�
n/a�
�
Disinfectant Wipes�
Disinfectant Wipes�
National Towelette Co.�
�
�
�
�
�
�
Dry Erase Markers�
Dry Erase Markers�
Sanford�
�
�
�
�
�
�
Dust Mop Treatment�
Dust Cleaner�
Ruchester Midland�
�
�
�
�
�
�
Emergency Stain Stick�
Stain Remover�
Oxiout�
�
�
�
�
�
�
Frebreeze�
Fabric Refresher�
Proctor & Gamble�
�
�
�
�
�
�
Germx�
Hand Sanitizer�
Vi-Jon�
�
�
�
�
�
�
Glass Wool�
Glass Wool�
Hach�
0/0/2009�
0�
0�
0�
n/a�
�
Glue Stick�
Glue�
Swinton Avenue Trading�
�
�
Goof Off�
Gunk Remover�
Valspor Cord�
�
�
Hydrogen Peroxide�
Hydrogen Peroxide�
Aaron Industries Co.�
�
�
Lysol Disinfectant Spray�
Disinfectant Spray�
Lysol�
�
�
Permanent Marker�
Permanent Marker�
Sharpie�
�
�
Spray Mount�
Spray Mount�
3M�
�
�
Spray Paint/Marker�
Marker�
Seymor�
�
�
Sun Dish Detergent�
Dish Detergent�
Sun�
�
�
Static Guard�
Anticling Spray�
Alberto-Culver USA�
�
�
Whiteboard Cleaner Spray & Wipes�
Whiteboard Cleaner�
Sanford�
�
�
White Out�
correction liquids�
Sanford�
�
�
409 Cleanser�
Cleanser�
Clorox�
�
�






�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�
�






Hazard Communication Chemical Inventory


Town of Marana Inventoried as of December 9, 2013


Town Wide (Organized by Department)





NFPA 704 Label





Department


(Storage Location)





Chemical/Product Name





Common Name





Manufacturer





SDS Rev.


Date	H	F	R	PPE








PAGE  

